INF2004 Embedded Systems Programming

Week 8 Tribunal 1

Architecture Defence & Integration Readiness | AI ideation, live update, AI-vetted gap check

| Item | Details |
| --- | --- |
| Purpose | Defend Week 6 architecture, justify changes, and show readiness for partial integration. |
| Core workflow | Pre-submission AI ideation -> live Zoom-updated version -> final AI-vetted gap check |
| Assessment focus | Evidence, engineering judgement, negotiation, and ability to identify missing work |
| Submission format | Completed document plus Zoom recording and evidence links |

Important: this submission has THREE versions inside one document - Version A: Pre-tribunal AI ideation. Each buddy uses AI to ideate their subsystem position before Zoom. - Version B: Live Zoom-updated version. Design decisions are updated during the Zoom discussion through challenge, negotiation, and constraint response. - Version C: Final AI-vetted gap check. After discussion, AI is used as a reviewer to identify missing assumptions, risks, tests, or unclear decisions. - Marks are not awarded for blindly following AI. Marks are awarded for critical evaluation and human engineering judgement; meaning, I want to see you guys discuss during zoom.

# Team Information

| Item | Details |
| --- | --- |
| Group ID | CSC14 |
| Repository Link | https://github.com/vanessayee/Embedded-System-Group-14-Project |

| Buddy | Student ID | Primary subsystem / persona |
| --- | --- | --- |
| Buddy 1 | 2501573 | Network Gatekeeper: WiFi, MQTT, telemetry |
| Buddy 2 | 2502591 | Real-Time Taskmaster: motors, encoders, PID |
| Buddy 3 | 2501720 | Track Purist: IR sensors, line following, barcode |
| Buddy 4 | 2500320 | Physics Watchdog: IMU, terrain, hump/motion events |
| Buddy 5 | 2502075 | Spatial Optimizer: ultrasonic, servo, obstacle profiling |

# Version A: Pre-Tribunal AI Ideation

Instructions • Each buddy must use AI before Zoom session to prepare subsystem-specific arguments, risks, and negotiation points. • The AI output must be critiqued. Identify what is useful, unrealistic, incomplete, or requires validation. • For Tribunal 1, compare against the Week 6 architecture freeze. For Tribunal 2, compare against actual integration evidence.

| Buddy | AI prompt used | Useful AI ideas | Weak / unrealistic AI ideas | Position before Zoom |
| --- | --- | --- | --- | --- |
| Buddy 1 | Please look at my week 6 Buddy 1 WiFi/MQTT and telemetry architecture, including the separate communication and collection of tasks, network-failure handling, shared resources, subsystem interfaces, and untested assumptions. Tell me if there is any form of disadvantages or unrealised potential issue in this design for buddy 1 | AI highlighted that separate tasks alone do not guarantee non-blocking operation. It identified the need to define telemetry queue capacity and overflow handling, agree telemetry fields with other buddies, and clarify how validated remote commands reach Motion Control. It also suggested testing disconnection and reconnection while monitoring local control-task timing. | The proposed interface and buffering suggestions are still design options. AI could not determine suitable queue sizes, retry intervals or timing performance without implementation details and measurements. These must be agreed and tested rather than treated as validated solutions. | I propose to retain separate Telemetry Collection and WiFi/MQTT Communication tasks, with local motion and safety independent of network availability. Implementation has not started yet for buddy 1 , so assumption A1 remains open. During Zoom, I want to try agree on telemetry fields, queue overflow behaviour and supervisory-command rules with the team. |
| Buddy 2 | Review the Week 6 Motion Control architecture: PID timing, encoder ISR load, stop()/turn accuracy, command arbitration, Fault handling and interfaces with Buddies 3, 4, 5. | AI identified that the 300 ms debounce could not validate recovery from a stall. A stall is detected as “PWM applied but no encoder ticks”, and in the Fault state the motors are off, so the condition clears by itself. The robot could repeatedly cycle through Fault, Idle, a new command and Fault again. Suggested fix: add a retry counter that escalates to the system-level Safe Stop, and define the stall criterion numerically. Motion Control only checks its queue every 20–50 ms, and a stop request could wait behind other queued commands, using up most of the 100 ms limit in NFR6. Suggested fix: wake the task from an event flag for stop and fault requests, let stop bypass the queue, and fix the task period at a single value (e.g. 20 ms) instead of a range. Adding motor current sensing for stall detection. This hardware is not available on the robot, so stall detection has to rely on encoder ticks. Adding a Safety Supervisor or Vehicle Controller task. This contradicts the team’s Section 4 decision to coordinate through queues and events, and it would add another task competing for CPU time. I will defend a single High-priority Motion Control Task as the sole owner of the motors, the creep speed design that reduces dead time between manoeuvres and improves completion time, and local automatic fault recovery, since motion and safety functions cannot depend on remote commands. I concede and will fix the following gaps found in the review: a single fixed task period, event-driven handling of stop and fault requests, a done / aborted / fault status in the motion-complete notification, a fault retry limit that escalates to Safe Stop, and a minimum creep speed set from bench measurements. I will also ask Buddy 3 to confirm the steering interface and that line following pauses during barcode actions, Buddy 5 to wait on the completion status and share the GPIO interrupt handler, and Buddy 4 to define the collision-to-stop path 10–15% of cruise speed may fall below the motor’s minimum reliable speed. Hence will need to define creep speed as the larger of 15% of cruise and the minimum controllable speed, measured on the bench. Needed to report done, aborted or fault as the Motion Complete notification has no fault or abort result, so Buddy 5 would wait until the recovery timeout if Motion Control faulted mid-move. | Adding motor current sensing for stall detection. This hardware is not available on the robot, so stall detection has to rely on encoder ticks. Adding a Safety Supervisor or Vehicle Controller task. This contradicts the team’s Section 4 decision to coordinate through queues and events, and it would add another task competing for CPU time. | I will defend a single High-priority Motion Control Task as the sole owner of the motors, the creep-speed design that reduces dead time between manoeuvres and improves completion time, and local automatic fault recovery, since motion and safety functions cannot depend on remote commands. I concede and will fix the following gaps found in the review: a single fixed task period, event-driven handling of stop and fault requests, a done / aborted / fault status in the motion-complete notification, a fault retry limit that escalates to Safe Stop, and a minimum creep speed set from bench measurements. I will also ask Buddy 3 to confirm the steering interface and that line following pauses during barcode actions, Buddy 5 to wait on the completion status and share the GPIO interrupt handler, and Buddy 4 to define the collision-to-stop path |
| Buddy 3 | Review my Week 6 Buddy 3 design for the Pico car using micro-TKernel. Check digital IR sensors for line following and barcode reading, task timing, pin mapping, and integration with Motion Control and obstacle recovery. T Kernel. Check digital IR sensors for line following and barcode reading, task timing, pin mapping, and integration with Motion Control and obstacle recovery. | The review highlighted that the normal 20–50 ms IR sampling interval may miss barcode edges while the car is moving. It suggested using an interrupt on IR_RIGHT D0 to record edge timestamps in a buffer for the Barcode Decoding Task to process. It also reminded me to confirm which sensor reads the barcodes and to verify the D0 pins. | The review's speed estimates (millimetres to centimetres per sample) have not been tested on our car, so they should not be treated as confirmed results. The interrupt approach may also interfere with encoder interrupts, and the buffer could overflow. The review's speed estimates (millimetres to centimetres per sample) have not been tested on our car, so they should not be treated as confirmed results. The interrupt approach may also interfere with encoder interrupts, and the buffer could overflow. | I will keep Line Following and Barcode Decoding as separate tasks. Line Following sends steering requests to Motion Control, while barcode edges are timestamped by an interrupt on IR_RIGHT D0 and decoded separately (a change from Week 6). I still need to verify the pin wiring and test barcode reading at different speeds. If the interrupt is unreliable, I will fall back to a 1 ms timer poll and agree this with the team. |
| Buddy 4 | Review my Week 6 Buddy 4 design for the Pico car using micro-T- Kernel. Check the IMU Monitoring Task (10 ms, High) and Hump & Terrain Analysis Task, IMU calibration and filtering, hump height estimation, tilt, collision and turn-rate detection, I2C use on GP16/GP17, fault handling, and integration with Motion Control and Telemetry. | The review suggested estimating hump height from the pitch angle combined with the distance travelled from the encoders, instead of from raw acceleration. It recommended a simple complementary filter and thresholds with a minimum duration to reduce false hump and collision detections caused by vibration. It also reminded me to keep the car still during calibration and to measure how long each I2C read takes so the 10 ms period is not missed. | Double integrating acceleration to get hump height drifts quickly and is not realistic on our car. A full Kalman filter is too heavy for this stage. Using the IMU to correct Motion Control turns would add coupling with Buddy 2 that was not in Week 6. The suggested thresholds and timing figures have not been tested on our car, so they are not confirmed. | I will keep the two-task structure from Week 6: IMU Monitoring runs every 10 ms and is the only task that uses the IMU on I2C0, and Hump & Terrain Analysis processes hump events. Buddy 4 only reports tilt, hump, collision and turn-rate events and never sends motor commands. Hump height will be estimated from pitch angle and encoder distance (a change from Week 6), so I need an agreedgreed distance value from Buddy 2. I still need to confirm which sensors the IMU module provides, set thresholds from test logs on the car, and agree with the team what the system does after a collision event. |
| Buddy 5 | Review the design review for buddy 5 ultrasonic and servobased obstacle architecture. Evaluate Obstacle Scan, Obstacle Profile and Avoidance & Recovery tasks, coarse/fine scanning, sensor accuracy, RTOS timing, fault handling and interaction with Buddy 2 and Buddy 3. | Separate scanning, profiling and avoidance responsibilities. Use explicit scan completion and failure events. Send high-level movement requests to Motion Control and use Buddy 3 information for line reacquisition. | Assuming ultrasonic scanning determines exact obstacle geometry. Adding unsupported sensors. Assuming separate tasks are always more efficient, or that scan timing and queue capacity need no validation. | I will keep the suggestion to separate the task architecture as obstacle scanning involves sensor acquisition and servo movement, profiling involves processing measurements, and avoidance requires coordination with Motion Control and Line Following. Separating these responsibilities allows each stage to expose completion, validity and failure |

# Week 6 Architecture Defence Snapshot Before Zoom

Select only 5 to 7 important Week 6 architecture claims that affect integration readiness. Do not repeat the entire Week 6 document. Each row should defend one claim using evidence, remaining weakness, and the buddy responsible. One smart-toaster example is shown; replace it with your own project claim.

| Week 6 artefact | What the team claimed in Week 6 | Evidence available now | Still weak or untested | Buddy defending it |
| --- | --- | --- | --- | --- |
| Section 3, Section 4, Section 6.5 | Obstacle handling is separated into Obstacle Scan, Obstacle Profile and Avoidance & Recovery. The subsystem sends high-level commands to Motion Control. | RTOS Task Table, Task Interaction Diagram, Buddy 5 Statechart, FR8–FR12. | Actual scan duration, profiling accuracy, CPU overhead, queue latency and recovery timeout are untested. | Buddy 5 |
| Section 4, Section 5 | Movement requests are coordinated through shared task events, system state and prioritised commands. Obstacle avoidance suspends normal navigation, with Safe Stop/Fault used when recovery fails. | Task Interaction Diagram, System-Level Statechart, Buddy 2/Buddy 5 statecharts and FR12/FR15. | Exact arbitration mechanism, event/queue ownership, stale command handling and full-system recovery need implementation and integration tests. | Buddy 2 Buddy 3 Buddy 5 |
| Section 3, Section 4, Section 6.3 | The Line Following Task reads the three digital IR sensors and sends steering requests to Motion Control. Barcode reading is handled by a separate Barcode Decoding Task. | The Week 6 task table, task interaction diagram, Buddy 3 statechart and Barcode & Line Specification. | The 20–50 ms sampling may be too slow for barcode reading while moving. The edge-capture idea still needs testing on the actual car. | Buddy 3 |
| Section 4, Section 6.3, Section 6.5 | Line Following reports when the line is lost or found again so Buddy 5 can use this during obstacle recovery. Steering should stop while the car is avoiding an obstacle. | Task interaction diagram, statecharts and FR11/FR12. | The team has not clearly decided who owns the overall system state. Without this, Line Following might keep sending steering requests during avoidance. | Buddy 3 Buddy 5 |
| Section 3, Section 4, Section 6.1, NFR9–NFR11 | Telemetry Collection and WiFi/MQTT Communication are separate tasks connected through a telemetry queue. Network or server failure must not prevent local motion and safety functions, and reconnection must use timed, non-blocking retries. | The Week 6 RTOS Task Table, Task Interaction Diagram and communication statechart document the intended separation and recovery behaviour. Assumption A1 identifies the required validation. No implementation or test evidence is available yet. | Task timing, network-call blocking behavior, queue capacity and overflow handling remain untested. Disconnection and reconnection tests are needed to check whether local control stays responsive while telemetry accumulates. | Buddy 1 |
| Section 3, Section 4, Section 6.2, NFR6–NFR8, NFR12, A2–A3 | A single High-priority Motion Control Task is the sole owner of motor PWM and direction. It runs PID speed control from encoder feedback at a 20–50 ms period. When a command is already queued at target, it decelerates to a creep speed instead of stopping, and a local Fault state disables the motors and recovers automatically. | RTOS Task Table, Task Interaction Diagram, Motion Control statechart (6.2), pin layout, NFR6–NFR8 and NFR12, Assumptions A2 and A3. | The task period is not yet fixed to one value. Section 3 assumes round-robin scheduling between equal-priority High tasks, which may not hold on µT-Kernel (to check against the spec), so distinct priorities may be needed. The 100 ms stop() limit, including queue wait, is untested. The creep speed (10–15%) and 300 ms debounce are placeholders, and creep may fall below the motor’s minimum speed. The stall criterion is not defined numerically, there is no fault retry limit, and the motion-complete notification has no abort/fault status. Encoder resolution, straight-line accuracy (15 mm per metre) and turn accuracy (±10°) are unmeasured. | Buddy 2 |
| Section 3, Section 6.4, FR5, FR6, FR16–FR19 | IMU processing is split into a 10 ms High-priority IMU Monitoring Task that owns the IMU and an event-driven Medium-priority Hump & Terrain Analysis Task. The IMU is calibrated before monitoring starts, and invalid IMU data moves the subsystem to Fault instead of being used. | RTOS Task Table, hardware pin layout (IMU on I2C0, GP16/GP17), Buddy 4 statechart, FR5, FR6, FR15–FR19, NFR13 and NFR14. | I2C read time within the 10 ms period, detection thresholds, hump height accuracy and false collision detections from vibration or hard braking are untested. What the system does after a collision event is not yet defined. | Buddy 4 |

## Shared Resource Allocation Matrix

Enter at least 3 shared resources, or more if the project has additional shared buses, timers, interrupts, actuators, sensors, queues, event groups, or shared data. A resource is “shared” if more than one task, module, interrupt, or buddy can affect it.

| Shared resource | Used by task(s) | Owner task / module | Access type | Contention risk | Mitigation | Evidence needed |
| --- | --- | --- | --- | --- | --- | --- |
| Motor command queue | Motion Control, Line Following, Barcode Decoding, Avoidance & Recovery | Motion Control — Buddy 2 | Multiple command producers, single consumer | Conflicting navigation, avoidance and stop requests | Define command priority and arbitration; safety stop overrides normal commands; reject stale commands | Concurrent-command logs and stop-latency test |
| Line status / recovery events | Line Following and Avoidance & Recovery | Line Following — Buddy 3 | Shared status or event notification | Buddy 5 receives stale status or misses line-reacquisition events | Defined line-status interface with validity and notification handling | Line-loss and reacquisition event logs |
| Obstacle scan sample queue | Obstacle Scan and Obstacle Profile | Obstacle Scan — Buddy 5 | Producer/consumer queue | Queue overflow, incomplete profiles or processing delays | Bounded queue, scan identifiers, validity checks and timeout handling | Queue usage, scan completion and overflow tests |
| Robot operating state / safety events | Motion Control, navigation, avoidance, IMU and communication tasks | Team-defined system-state interface | Multiple readers and event producers | Inconsistent state changes or conflicting recovery decisions | Define authoritative state ownership, prioritised events and safe-state transitions | Multi-task fault and recovery logs |
| Telemetry queue / shared status snapshots | Telemetry Collection, WiFi/MQTT Communication and sensor/motion tasks | Telemetry Collection / Communication — Buddy 1 | Multiple data producers and network consumer | Slow network publishing blocks tasks or overfills the queue | Bounded buffering, non-blocking transfers and handling for stale/non-critical telemetry | Queue-depth, publishing-rate and disconnection tests |
| IR_RIGHT D0 (GP27) | Line Following Task and barcode edge-capture interrupt | Line Following — Buddy 3 | Level read by the task plus edge interrupt on the same pin | Edge interrupts add load or chatter on a pin also used for steering; if the sensor is offset for barcodes it may not see the line | Interrupt only timestamps edges; Line Following only reads the pin level; decide IR_RIGHT's role in steering | Edge log while following a line and crossing a barcode; sensor position check on the car |
| GPIO interrupt (shared by encoders and barcode) | Encoder ISR (GP0, GP1, GP7, GP28) and barcode edge-capture ISR (GP27) | Shared — Buddy 2 (encoders) and Buddy 3 (barcode) | Interrupt handlers sharing one GPIO interrupt | Encoder edge counting delays barcode timestamps, or the reverse, causing late or missed edges at high speed | Keep every handler to counting or timestamping only; estimate the combined edge rate at maximum speed | Edge-rate calculation and a maximum-speed test with missing-edge and overflow counts |
| CPU time and RTOS scheduling | Motion Control, Line Following, IMU Monitoring, Obstacle Scan, Avoidance & Recovery | Shared — priorities agreed by all buddies | Shared processor time between equal-priority tasks | Five High tasks, no guaranteed time-slicing, so a long task could delay the motor PID loop; 10 ms IMU I2C transfers could too | Distinct priorities, fixed periods, no long operations in High tasks, measure worst-case run times | Timing log of period, jitter and worst-case run time with all tasks running |
| Processed IMU data / IMU event queue | IMU Monitoring, Hump & Terrain Analysis and Telemetry Collection | IMU Monitoring — Buddy 4 | Single producer, multiple consumers | Hump analysis or telemetry reads a half-updated or stale sample; hump or collision events are lost if the queue fills | Bounded event queue; latest cleaned sample shared as a timestamped snapshot with a validity flag; consumers ignore invalid or stale samples | Queue-depth and dropped-event counters during a hump run; telemetry log showing timestamps and validity flags |

## Subsystem Interface and Code Structure Map

Enter at least one interface or module per buddy. Each row should identify who provides the module, who consumes it, what inputs and outputs it exposes, and how it will be validated.

| Module / file / interface | Provider | Consumer(s) | Purpose | Inputs | Outputs | Interface rule | Validation rule |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Line Following Task | Buddy 3 | Motion Control; Avoidance & Recovery | Read the three digital IR sensors to follow the line and detect when the line is lost or found again. | Digital D0 readings from IR_LEFT, IR_CENTER and IR_RIGHT, plus the agreed navigation mode. | Steering request to Motion Control; line status (on line / lost / reacquired) with timestamp | Does not control motor PWM. Sends steering requests to Motion Control only during Normal Navigation. | Test straight lines, curves, line loss and recovery. |
| Barcode edge capture and ring buffer (IR_RIGHT D0) | Buddy 3 | Barcode Decoding Task | Record edge times from IR_RIGHT D0 while the car crosses a barcode. | Digital edge signal from IR_RIGHT D0 and a timer; pin and interrupt behaviour still need checking. | Buffered edge timestamps and an overflow flag for Barcode Decoding Task | The interrupt handler only records edge times. It does not decode the barcode or control the motors. Reject incomplete readings. | Test barcode reading at different speeds and check for missing edges or buffer overflow. |
| Barcode Decoding Task | Buddy 3 | Motion Control (through the command queue) | Use the saved edge timings to decode the barcode. | Buffered edge timings from IR_RIGHT D0. | Decoded command or invalid result | Does not read the sensor directly or control the motors. An invalid barcode must not produce a command. | Test known and invalid barcodes and check that only valid readings produce a command. |
| Motion Control Task and motion command interface | Buddy 2 | Line Following Task; Barcode Decoding Task (navigation commands); Avoidance & Recovery Task; WiFi/MQTT Communication Task (supervisory stop/mode requests); Telemetry Collection Task | Execute move, turn, steering and stop requests using encoder feedback, and report the result to the requesting task. | Prioritised motion requests: line/steering, barcode navigation, obstacle avoidance and supervisory stop/mode; wheel speed and distance from the encoder module; Fault flag; current system mode (Normal Navigation / Navigation Action / Obstacle Handling / Safe Stop) from the team-defined state interface. | Motor speed and direction targets for the motor driver; motion status (Idle / Moving Straight / Turning / Fault); motion-complete result (done / aborted / fault); speed, distance and state for telemetry. | Sole owner of motor PWM and direction, and no other module writes motor outputs. Command priority is Safe Stop/Fault > Obstacle Avoidance > Barcode Navigation > Normal Line Following. Steering requests are accepted only during Normal Navigation. stop() bypasses the queue, flushes queued commands and takes effect within 100 ms. Invalid or out-of-range requests are rejected. Fault disables motor output. | Test move distance and turn angle, stop() latency, and simultaneous commands from several tasks to check priority and queue flushing. Inject encoder faults and stalls to check the fault state, retry limit and the aborted/fault result. |
| Encoder ISR and tick counter | Buddy 2 | Motion Control Task; Telemetry Collection Task (through motion status) | Count wheel encoder edges for the left and right wheels and provide tick counts for speed and distance estimation. | Encoder A/B edge signals: left on GP0/GP1 and right on GP7/GP28. | Tick count and direction per wheel; encoder fault flag (no ticks while motors are driven). | The interrupt handler only updates counters, with no PID, motor control or blocking calls. Counts are read atomically by Motion Control. The shared GPIO interrupt handler must be agreed with the other buddies, because the RP2040 allows one GPIO callback per core. Left encoder on GP0/GP1 (default UART0 pins), so debug output must use USB, not UART0 (to confirm on the board). The right encoder on GP28 may be affected by the VBAT jumper position; to verify against the Robo Pico schematic and on the board. | Rotate each wheel a known number of turns and compare counts. Run at creep, mid and maximum speed and check for missed or noisy counts. Confirm GP28 and GP0/GP1 give clean edges with USB-only debug output running, and with the VBAT jumper in the position the schematic requires. |
| Motor driver and PID speed control | Buddy 2 | Motion Control Task | Convert wheel speed targets into PWM and direction signals for the on-board motor driver and hold the target speed with PID control. | Left and right wheel speed targets; measured speed from the encoder module; Fault/stop request. | PWM duty and direction on GP8/GP9 (left motor) and GP10/GP11 (right motor). | The only module that writes motor pins. Output is limited to a safe PWM range, and PWM is forced to zero in Fault and on stop(). PID runs inside the Motion Control Task period, not in an interrupt. | Run step-response and PID tuning tests at several speeds and check straight-line deviation. Check that stop() and Fault force PWM to zero. |
| Obstacle profile | Buddy 5 | Avoidance & Recovery | Convert scan measurements into obstacle position, width and clearance estimates | Validated ultrasonic angle-distance samples | Obstacle profile, validity status | Invalid or incomplete scan data must not produce a trusted avoidance profile | Test profiles against known obstacle dimensions and invalid samples |
| Avoidance & recovery | Buddy 5 | Motion Control, Line Following | Select avoidance actions and coordinate line reacquisition | Obstacle profile, motion completion and line status | High-level movement requests, recovery/fault events | Buddy 5 does not control motor PWM directly; line recovery uses Buddy 3 status | Obstacle detection, avoidance, line recovery and failed-recovery tests |
| Obstacle scanning | Buddy 5 | Obstacle Profile | Control servo scanning and acquire ultrasonic samples | Scan request, angle range, distance measurements | Angle-distance samples, scan completion or error status | Only Obstacle Scan controls scanning hardware; profiling consumes scan results | Compare measurements with known distances and angles; verify timeout handling |
| Telemetry Collection Task | Buddy 1 | WiFi/MQTT Communication Task | Collect robot status and measurements and prepare telemetry messages. | Status and measurements from Buddies 2–5, including validity and timestamps where available. | Formatted telemetry messages placed in the telemetry queue. | Read data through agreed subsystem interfaces. Collection and queue submission must not block time-critical control tasks. Queue capacity and overflow policy must be agreed. | Supply known subsystem values and check message fields, units and timestamps. Fill the queue and check that control tasks remain responsive. |
| WiFi and MQTT Communication Task | Buddy 1 | MQTT server; Motion Control for accepted supervisory requests | Maintain connections, publish telemetry and heartbeat, and validate incoming commands. | Telemetry queue, network configuration and incoming MQTT commands. | Published messages, connection status, acknowledgements and validated command requests. | Own the WiFi/MQTT connection. Reject invalid commands and pass accepted requests through the agreed command interface. Do not write motor PWM directly. Reconnection must not block local control. | Test publishing, malformed commands and WiFi/broker disconnection and recovery while checking local task timing. |
| IMU Monitoring Task | Buddy 4 | Hump & Terrain Analysis Telemetry Collection | Read, calibrate and filter IMU data every 10 ms and detect tilt, turn rate, motion state and collisions. | Accelerometer and gyroscope readings over I2C0 (GP16GP17),calibration offsets. | Cleaned IMU sample with timestamp and validity flag, tilt, turn rate, motion state and collision events. | Only this task accesses the IMU. Does not send motor commands. Invalid or uncalibrated data is flagged and never passed on as valid. | Stationary, driving, turning and impact tests measure task period and I2C read time unplugged IMU fault test. |
| Hump & Terrain Analysis Task | Buddy 4 | Telemetry Collection (Buddy 1) | Detect humps, estimate each hump height and keep the highest hump of the run. | Validated pitch and acceleration samples from IMU Monitoring encoder distance from Motion Control. | Hump detected events, hump height, and highest hump peak. | Uses only validated IMU samples. Does not access the IMU or encoder hardware directly and does not affect motion. | Drive over a hump of known height at different speeds and compare estimated and measured height check no humps are reported on flat track or curves. |

## Week 6 Assumption Closure

Enter at least one assumption per buddy, preferably copied from Week 6. Each row should show whether the assumption is still open, validated, deferred, or converted into a risk.

| Week 6 assumption ID | Original assumption | Current status | Evidence collected | Remaining risk | Next action |
| --- | --- | --- | --- | --- | --- |
| A1 – Buddy 1 | The Pico W can periodically publish MQTT telemetry and heartbeat messages, and recover from WiFi or MQTT broker disconnection, without blocking or delaying time-critical motion, navigation and safety tasks. | Needs validation | The Week 6 task table and communication statechart document the proposed approach. No implementation or timing measurements are available yet. | Blocking network operations or repeated reconnection attempts could delay control tasks, overflow the telemetry queue and produce stale or lost telemetry. | Publish representative telemetry while simulated or actual control tasks run. Disconnect and reconnect WiFi and the broker, then measure control task timing, queue usage, message loss and reconnection time. Keep A1 open until the results support it. |
| A2 – Buddy 2 | The robot can reliably decelerate to the fixed creep speed within a short, predictable distance before a queued turn begins, without overshooting into the turn at too high a speed. | Needs validation | The creep-speed requirement (NFR12) and the Motion Control statechart document the intended approach. No physical deceleration measurements are available yet. | The 10–15% creep speed and 300 ms debounce are placeholders. Creep speed may fall below the motors' minimum reliable speed, and the deceleration distance has not been measured. If it varies, the turn angle calibration is not valid. | Measure the minimum controllable motor speed. Set creep speed as the larger of 15% of cruise speed and that minimum. Bench-test the deceleration profile at several starting speeds and record the distance needed. Then check the turn angle against NFR8. Keep open until the results are repeatable. |
| A3 – Buddy 2 | Wheel encoders give reliable, sufficiently fine resolution quadrature signals across the robot's full speed range, including the low creep speed used before turns. | Needs validation | The Week 6 pin layout (left encoder GP0/GP1, right encoder GP7/GP28 with interrupts), the Encoder ISR / Tick Counter in the Task Interaction Diagram and the Motion Control statechart document the intended approach. No bench measurements are available yet. | Encoder resolution (ticks per wheel revolution) has not been measured, and it is not yet confirmed that the encoders give true A/B quadrature, which direction sensing for reverse moves depends on. At creep speed there may be only 0–2 ticks per 20 ms period, which would give noisy speed estimates and could trigger false faults. The RP2040 allows only one GPIO interrupt callback per core, so the encoder interrupts could clash with other buddies’ pins. The left encoder uses GP0/GP1, which are the default UART0 pins, so debug output must use USB, not UART0 (to confirm on the board). The right encoder uses GP28, which may be affected by the VBAT jumper position (to verify against the Robo Pico schematic). | Rotate each wheel a known number of turns and compare the counts. Run at creep, mid and maximum speed and log missed or noisy counts. Confirm the encoder type and agree a single shared GPIO interrupt handler with Buddies 3 and 5. Check the Robo Pico schematic and the board for the VBAT jumper position and confirm GP28 reads clean encoder edges. Configure stdio over USB only and confirm GP0/GP1 give clean edges with debug output running. Keep open until the counts match the measured distance. |
| A4 — Buddy 3 | The digital IR sensors can tell the difference between the barcode and a normal line or curve. | Needs validation | The specification gives the line and barcode dimensions, but we have not confirmed the sensor readings on the moving car. | A curve might be mistaken for a barcode, or the barcode might not be read correctly. | Test the line, curves and barcode at different speeds. Keep this open until the readings are reliable. |
| A5 — Buddy 4 | The IMU can provide stable and accurate enough acceleration and gyroscope readings for detecting humps, tilt, collisions and turn rate. | Needs validation | FR5, FR6, FR16, FR19, NFR13, NFR14, the IMU pin layout and the Buddy 4 statechart document the approach. No stationary, driving or hump test logs collected yet. | Vibration and motor noise could cause false hump or collision events, and hump height may be inaccurate. | Log stationary and driving data, drive over a known hump and do controlled turns. Set thresholds from the logs and keep this open until detections are reliable. |
| A6 — Buddy 5 | Ultrasonic measurements are stable over the required detection range, and servo positioning is repeatable during coarse and fine scans. | Needs validation | FR8–FR12, ultrasonic/servo pin layout, Buddy 5 task structure and statechart document the intended approach. No physical scan measurements supplied. | Noisy distances or inaccurate servo positioning may generate incorrect profiles and unsafe avoidance decisions. | Scan known obstacles at several distances and angles. Compare estimated boundaries with physical measurements and record scan timing and repeatability. |

# Version B: Live Zoom-Updated Version

Instructions • During the recorded Zoom session, update this section live or immediately from discussion notes. • Record what changed from Version A and why. A change without justification is weak evidence. • Every buddy must contribute at least one challenge, defence, trade-off, or correction. • Any use of AI during the Zoom session must be screen-shared and visible to all participants at the point it is being discussed. • If AI-generated content is introduced during the live discussion, the team must explain the prompt used, the output received, and why the output is being considered. • AI suggestions must be challenged, validated, refined, or rejected through human engineering discussion; “AI said so” is not sufficient justification.

Acceptable evidence examples: GitHub commit link, short test log, video timestamp, oscilloscope or serial monitor screenshot, pin test result, task timing measurement, AI prompt and critique, meeting decision note, or photo of hardware wiring.

AI-use declaration: Do not ask AI to complete this template on behalf of the team. Use AI as a reviewer, challenger, or brainstorming partner. The team confirms that all final entries were reviewed, corrected, and accepted by the team, and that every member can explain the decisions related to their subsystem.

## Buddy Contributions During Zoom

Each buddy must complete one row. The contribution must be a real technical challenge, defence, trade-off, or correction raised during Zoom, not only an attendance statement.

| Buddy | Key challenge, concern, or argument raised | Challenge received | Decision or compromise reached |
| --- | --- | --- | --- |
| Buddy 1 | Key challenges will be handling of consistent interpretation of binary encoding and decoding throughout buddy 1 to 5 | How to ensure decoding and encoding of alphanumeric values are accurate throughout the entire system | To come out with a standard encoding and decoding standards to ensure information passed throughout the system is accurate |
| Buddy 2 | Minimize the bit representation of motor commands, encoder measurements and motion states while preserving motion accuracy, performance and safety | If packing motor commands and encoder data would cause overflow, incorrect decoding or race conditions, affecting motor control and the 100 ms stop requirement. | Retain Motion Control as the sole motor owner. Define minimum-bit representations based on valid ranges, use a shared fault bitmask and ensure safe access to packed data. Verify decoding correctness and control-task timing before integration. |
| Buddy 3 | To determine the minimum bits required for IR sensor states, navigation commands and status flag without losing information. | Ensuring that reducing the number of bits does not result in overlapping bit pattern, loss of valid states or incorrect interpretation of sensor reading and navigation commands. | Agree on a standardized data format and optimize bit allocation to represent all valid IR sensor states, barcode information, navigation commands, status flags. |
| Buddy 4 | The accuracy loss will be a trade off when compacting IMU values like hump height to compact it into fewer bits by changing the step size from 1 to 2 so essentially instead of 0-100mm which is 101 values, it becomes 0-100mm with 51 values. | Whether the saving of 1 bit from 7 bit to 6 bit is worth changing the step size from 1mm to 2mm as its doubling the rounding error to +-1 mm. | Keep 2mm steps which is 6 bits as the IMU and encoder based estimate is not accurate to 1mm anyways which the 1mm might be stored noise rather than actual accurate recording. |
| Buddy 5 | I propose keeping the separation of obstacle scan, obstacle profile, and avoidance & recovery task. However, the new bit requirement raises the concern to minimize the representation for the measurements, while preserving the precision/accuracy of the measurements. | Whether using bit-packing to transfer data between RTOS tasks would affect the accuracy/precision of the measurements, potentially leading to incorrect obstacle profiling and unsafe avoidance decision | To define the minimum-bit representation for buddy 5, use safe queue transfers and appropriate synchronization, and include verification that unpacked measurements match original values without compromising obstacle profiling or safety. |

## Live Decision Change Log

Enter at least 3 changes AND 3 confirmed no-change decisions from the Zoom discussion. Include the change type and evidence link or timestamp whenever possible. One row may state “No change” if the team defended the Week 6 design successfully.

| Original idea / Week 6 design | Updated decision | Change type | Reason for change | Evidence link / timestamp | Owner / follow-up |
| --- | --- | --- | --- | --- | --- |
| Buddy 5 keeps the separation of the task's obstacle scan, obstacle profile, and avoidance & recovery | Keep the separation of the task when implementing the bit requirement | No change | - | Time: 1312 19/10/2026 | Hai Seang (buddy 5) Follow up - identify all task bit requirements. |
| Buddy 4 has to have Hump & Terrain Analysis outputs such as hump height. | Hump height stored and sent as a scaled integer over 0-100mm in 2mm steps so that it reduces from 7 bits to 6bits or changes to 4mm steps which reduces 7 bits to 5 bits depending on the noise error range. | Simplified | Minimum bit change requirement, step size change does not affect as the it is matched to the realistic IMU and encoder error so the only thing that is removed is the the noise that is picked up. | Time: 1313 19/10/2026 | Buddy 4: confirm step size after known-hump test |
| Buddy 3 line following checks the sensors every 20-50 ms and passes barcode readings to decode. | Implement an interrupt driven approach for IR_RIGHT sensor (GP27) to capture black/white transitions in a buffer. The barcode decoding module processes the buffered readings asynchronously. For backup, retain 1 ms periodic sensor polling using timer. | Redesign | Checking every 20-50 ms can miss stripes when the car is moving, because the car travels a few millimeters to centimeters between checks. | Time: 1314 19/10/2026 | Buddy 3 Test the car over a barcode at different manually measured speed. (push with hands) |
| Buddy 1 Initially wanted information to be passed as text value over MQTT communication | MQTT information communication will now all be sent via binary bits format. Standards to encode and decode values will be enforced for buddy 1 – 5 communication | Redesign | Compression of application fields to fasten data size and improve correctness and performance of robot performance | Time: 1314 19/10/2026 | Jun Jiang (Buddy 1) - To ensure values that are sent over MQTT are read correctly when converted to binary format |
| Record hump data and identify the highest hump peak | Highest hump is still being tracked by the analysis where the calculation still stays full resolution inside the task and only compacted if the value is sent to the telemetry | No Change | - |  | Buddy 4 with 1: Agree bit layout in MQTT payload |

## Resource and Assumption Review

| Item challenged | Evidence presented | Decision made | Impact on other buddies |
| --- | --- | --- | --- |
| Bit packing measurements to transfer data between task may affect measurements precision/accuracy or data integrity | Week 6 Design review section 3 defines a scan sample queue between 2 tasks. The bit requirement mandates the bit representation for the inter-task messages. | Define the minimum bit representation based on the required measurement ranges and precision. Use safe queue transfers and verify that unpacked measurements data integrity. | Affect buddy 5 primarily. Buddy 1 might be affected for MQTT telemetry |
| IMU is accurate enough to detect humps, tilt, collisions and turn rate | Week 6 Design Review has information that Motor vibration makes IMU readings to be noisy while driving, and the gyroscope drifts. No test logs yet. | Need validation. Calibrate the car while it is still, filter the readings, and set thresholds from real tests. | False events would send wrong telemetry to Buddy 1 and Buddy 2 must keep the motors off during calibration. |
| Consistent interpretion of packed telemetry | Change requirement requires compact binary application data. Initial deisgn in week 6 have not yet specify field layout and scaling remain unspecified | Will Define a shared field specification and known-value encoding/decoding test before integration | Buddy 2-5 needs to provde application data field specification for MQTT Communication |
| Ownership of car’s operating mode | In the week 6 design review feedback identified that the system has no explicitly assigned owner for mission state. | Assign to buddy 2 because it owns the motion control, so it can ensure that steering commands are executed only when the current mode allows them. | Buddy 3, 4 and 5 send events to Buddy 2 (barcode command, line lost, collision, obstacle detected, avoidance complete, recovery failed). Buddy 2 updates the mode. Buddy 1 only reads the mode for telemetry, and Buddy 3 reads it to stop steering outside Normal Navigation. |

## Risk Assessment and Subsystem Test Plan

### Integration Risk Register

Enter at least 3 integration risks. Include at least one timing or scheduling risk, one hardware or wiring risk, and one interface or team-integration risk.

| Risk ID | Risk | Cause | Impact | Likelihood | Severity | Mitigation | Owner | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | Robot actions might be incorrect | Value of binary bits sent over MQTT could be encoded and decoded incorrectly | Incorrect behavior of robots might be observed | Medium | High | Ensure there are addition payload tags added for checking during transmission of data. Introduce sufficient error-handling if any incorrect actions are found, | Buddy 1 | Open |
| R2 | IMU Monitoring Tasks misses its 10 ms period | I2C reads, filtering and bit-packing take too long or other high-priority tasks delay it | Missed or late hump, tilt and collision detections | Medium | High | Measure task run time and period jitter, keep packing code short and only IMU monitoring uses I2C0 | Buddy 4 | Open |
| R3 | Barcode changes are saved late or missed. | The wheel encoders and the IR sensor sharing one interrupt and changes come very fast at high speed. | Barcode is misread or skipped, so the car may turns wrongly or ignore it. | Medium | Medium | Keep a shared interrupt handler lightweight by recording encoder counts and barcode timestamps only, | Buddy 3 | Open |
| R4 | Loss of measurement/inaccurate measurements | Insufficient bit, or incorrect scaling/ shifting/ rounding | Incorrect obstacle profiles and unsafe avoidance decisions | medium | high | Define required measurements ranges and precision and verify the values for both pack and unpack data | Buddy 5 | Open |

### Subsystem Test Plan

Enter at least 4 cross-subsystem tests. Each test must involve the work of at least two buddies and must include a success criterion and evidence type, such as a video, serial log, screenshot, timing result, or commit link. At least TWO test should involve timing, safety, or shared-resource concern.

| Subsystem | Test case | Success criteria | Evidence required | Owner | Status |
| --- | --- | --- | --- | --- | --- |
| Avoidance & Recovery & Motion Control | Transfer bit-packed avoidance commands and motion results between avoidance & recovery and motion control | All valid commands and completion/faults outcomes decode correctly including minimum and maximum permitted fields values | Serial logs showing original, packed and unpacked values. | Buddy 2 & 5 | Open |
| Hump & terrain analysis + motion control + telemetry | Drive over a hump of known height 5 times | Estimate height within +-4 mm of the real height, dashboard should show the same value the PICO sent | Video, serial log of pitch, distance and height and dashboard screenshot | Buddy 1, 2, 4 | Open |
| Binary data interfaces and MQTT telemetry across Buddies 1–5 | Pass known motion/encoder values (Buddy 2), line status and barcode results (Buddy 3), hump/tilt/collision data (Buddy 4), and obstacle/avoidance/recovery status (Buddy 5) through their agreed interfaces to Buddy 1 and the MQTT/Node-RED decoder. Test minimum, maximum, invalid and scaled values, plus combinations of fault flags. | Integer values, state codes and fault flags decode exactly as specified. Measurements retain the agreed units, sign and precision. Invalid or unavailable data is flagged correctly. Malformed payloads are rejected without crashing. Results match independently calculated expected values. | Test table showing original inputs, expected encoded bytes, actual payload bytes and decoded outputs; subsystem and Node-RED logs. | Buddy 1 coordinates; Buddies 2–5 provide test values and verify their data. | Open |
| Barcode detection at different speeds | Drive over a known barcode at low, medium and high speeds with recording sensor transitions. | The correct barcode is ended in 15 consecutive trials at each tested speed, without missed or duplicated transitions. | Serial log | Buddy 3 | Open |

## Constraint Injection / Curveball Response

Instructions • Record the injected constraint exactly as given. • Explain how it affects all five buddies, even if the impact differs by subsystem. • A good response shows prioritisation, trade-off reasoning, fallback behaviour, and evidence-aware decision making.

| Injected constraint | Buddy affected | Immediate impact | Adaptation / compromise |
| --- | --- | --- | --- |
| The robot car firmware must minimize the number of bits used to represent application data. | Buddy 1 | The planned text/JSON MQTT application payloads must change to compact binary fields. Telemetry queues and command messages also need suitable packed representations. Incorrect field mappings, units or scaling could cause the firmware and server to interpret data differently. | Agree field widths, ranges, units, scaling and state/fault codes with Buddies 2–5. Use shared encoding/decoding functions and update Node-RED to decode the binary payloads. Retain separate telemetry and communication tasks. Accept additional encoding and debugging complexity in exchange for smaller messages, while preserving required measurement precision. Validate with known-value tests and before/after message-size, RAM and timing measurements. |
| The robot car firmware must minimize the number of bits used to represent application data. | Buddy 2 | Motion commands, encoder counts, PID-related application data, motion states should use the minimum required bit representation. Bit packing may introduce overflow, incorrect decoding, synchronization issues and processing overhead, potentially affecting motor accuracy, task timing and safety. | Motion control remains the owner of the motor output and PID processing. Define minimum bit representation ranges. Combine fault conditions into a shared bitmask and use atomic access or synchronization for shared data. |
| The robot car firmware must minimize the number of bits used to represent application data. | Buddy 3 | IR sensor states and barcode related data must use fewer bits without affecting line following or barcode detection. | Use 2-bit codes for up to 4 navigation commands, and allocate additional bits where required for IR sensor combinations and barcode data. |
| The robot car firmware must minimize the number of bits used to represent application data. | Buddy 5 | Measurements must use minimum number of bits required for their valid ranges. Bit-packing also introduce concerns about precision of values and data integrity | Keep the separate 3 task while implementing the bit requirement. To define the bit widths for measurements and use safe queue transfers and appropriate synchronization, verify unpack and pack values. |
| The robot car firmware must minimize the number of bits used to represent application data. | Buddy 4 | Hump height, pitch, turn rate and IMU states can no longer be stored as float or int. Compacting them will keep the values +-1 which could reduce the hump height accuracy. | The calculation. Will be done at full resolution inside the IMU tasks and it will only be compacted when its being stored or send to other buddies. It becomes 2mm steps which is 6 bit instead of 7 when its being sent or stored. |

## Integration Readiness Self-Assessment

Complete all rows. Use Green only if there is evidence, Yellow if partly ready or untested, and Red if the area is unclear or blocking Week 10 integration.

| Readiness area | Status | Evidence | Concern |
| --- | --- | --- | --- |
| Hardware wiring known | Amber | Architecture in design review, feedback by prof has nothing critical related to hardware wiring | Wiring not yet verified on the car |
| RTOS task ownership clear | Green | Week 6 Section 3 identifies task owners. Version B assigns operating-mode ownership to Buddy 2 | Shared operating-state updates and transitions still require implementation and testing |
| Shared resources identified | Amber | Version A includes a shared resource allocation matrix covering command queues, interrupts, telemetry, CPU scheduling and sensor data. | Queue capacity, packed-data synchronization and command arbitration remain untested or insufficiently specified |
| Critical statecharts implementable | Amber | Week 6 Sections 5–6 provide system-level and subsystem statecharts. Version B defines additional interface and ownership decisions. | Ready/Wait-for-Start, Mission Complete, obstacle detection during turning and fault clearing remain unspecified |
| Safety or fault handling defined | Red | Week 6 FR12/FR15 define Safe Stop and invalid-data handling. The Motion Control interface establishes safety-command precedence. | Shared fault-mask handling, collision response, fault clearing and recovery rules not finalized. |
| Week 10 partial integration feasible | Amber | Responsibilities, architecture, interfaces and proposed cross-subsystem tests are documented. | Hardware verification, binary interfaces, timing, data integrity and actual integration yet to be tested |

Updated readiness position before the Week 10 partial integration demo:

The team has established the overall system architecture, with defined subsystem responsibilities, RTOS tasks, hardware pin mappings, task interactions, and statecharts from the Week 6 Design Review. These provide a foundation for Week 10 partial integration.

However, the new minimum-bit data representation requirement introduces additional implementation work across all five subsystems, including motion control, line and barcode sensing, IMU monitoring, obstacle handling, and MQTT communication. The team must define compact data structures, standardise inter-task message formats, and ensure safe access to shared packed data without compromising measurement precision, system functionality, or real-time performance.

Before Week 10, the team will prioritise implementing the agreed data representations, verifying hardware connections, establishing compatible subsystem interfaces, and conducting initial cross-subsystem integration tests. Remaining uncertainties include RAM usage, message-size reductions, task scheduling, data integrity, and fault-handling performance, which require further validation. Overall, the project is partially ready for integration, with key architectural decisions established but implementation and testing still in progress.

# Version C: Final AI-Vetted Gap Check

Instructions • After the live discussion, give the updated Version B content to AI and ask AI to act as a strict embedded systems reviewer. • Ask AI to identify missing assumptions, weak evidence, unclear interfaces, untested failure modes, and resource conflicts. • For every AI-identified gap, decide whether to fix, reject, or defer it, and explain why.

AI reviewer prompt used

Act as a strict embedded systems architecture reviewer for our INF2004 Tribunal 1 project. Review our updated Version B decisions against the Week 6 architecture and the new injected constraint: "The robot car firmware must minimize the number of bits used to represent application data." Our project uses a Raspberry Pi Pico-based robot car with micro-T-Kernel, involving five subsystems: - Buddy 1: WiFi, MQTT and telemetry. - Buddy 2: Motion Control, motors, PID and encoders. - Buddy 3: IR sensors, line following and barcode decoding. - Buddy 4: IMU monitoring and hump/terrain analysis. - Buddy 5: Ultrasonic scanning, obstacle profiling and avoidance/recovery. Review the team's decisions to retain subsystem task separation while implementing compact binary representations and packed inter-task messages. Identify the gaps in the report. For each identified gap, explain its impact, severity, recommended mitigation, required evidence and responsible buddy. Distinguish between architectural decisions already agreed upon and assumptions still requiring physical testing. Do not assume implementation or test evidence exists if it has not been provided. Prioritize issues that could prevent safe and reliable Week 10 partial integration.

| AI-identified gap | Team response | Action taken | Evidence / reason |
| --- | --- | --- | --- |
| Undefined binary field specifications | Agree on exact representations across all buddies. | Create a shared data representation table specifying range, bits, units, scaling and invalid codes. | Version B proposes binary communication, but complete field specifications are absent. |
| Race conditions in shared packed data | Synchronization is required. | Define ownership, atomic access or appropriate locking for shared bitfields, ISR data and queues. | The injected requirement explicitly prohibits unsafe read-modify-write access to shared packed values. |
| Loss of obstacle measurement precision | Preserve sufficient precision for safe avoidance. | Buddy 5 will define acceptable measurement ranges and resolution, then verify packing, unpacking and obstacle profile accuracy. | Risk 4 identifies incorrect scaling or rounding as a possible cause of unsafe avoidance. Assumption A6 remains unvalidated. |
| Incomplete shared fault handling | Define fault codes and response priority. | Agree on fault-bit allocation, fault ownership, Safe Stop conditions and interactions between Buddies 2, 4 and 5. | Version B states that collision responses and shared fault-mask handling are not finalized. |
| Unknown RTOS timing overhead | Ensure minimum-bit encoding does not affect deadlines. | Measure task execution time, jitter and queue latency with packing enabled and all relevant tasks active. | Version B Risk R2 identifies possible violations of Buddy 4's 10ms IMU task period. Motion Control timing is also unverified. |
| MQTT binary decoding compatibility | Establish matching encoder and decoder specifications. | Buddy 1 will coordinate known-value tests between Pico firmware and Node-RED, including invalid values, scaling and fault combinations. | Version B changes MQTT application fields from text/JSON representations to compact binary fields, but successful decoding is unverified. |
| Hardware and sensor validation | Defer closure until testing. | Verify wiring, encoder signals, ultrasonic accuracy, servo repeatability and IMU measurements on the physical car. | Week 6 Design review provides pin mappings and component architecture. |
| Memory savings are unproven | Quantify actual savings. | Compare actual RAM usage, C structure sizes and queue/MQTT message sizes before and after packing. | Smaller logical fields do not necessarily produce smaller structures because of storage-unit allocation, padding and alignment. Week 13 requires quantitative comparisons |

# 5. Final Tribunal Summary

| Final question | Team answer |
| --- | --- |
| What is the strongest part of the current design? | The strongest part of our design is the clear separation of RTOS task responsibilities across all five buddies. Each subsystem has defined ownership and communicates through task interfaces, queues and events, while Motion Control exclusively controls the motors. The architecture can accommodate the minimum-bit requirement without changing its fundamental task separation, supporting modular implementation and integration. |
| What is most likely to fail? | The inconsistent binary data representation and interpretation between subsystems. Incorrect field widths, scaling, packing or unpacking may corrupt sensor measurements and movement commands. Race conditions involving shared packed data, together with unverified RTOS timing, may also affect system reliability and safety. |
| What evidence supports readiness? | Our Week 6 Design Review provides architectural evidence through the RTOS task table, hardware pin mappings, task interaction diagram, subsystem statecharts and documented assumptions. Version B records the team's discussion and adaptation to the minimum-bit requirement, including planned interface changes and cross-subsystem tests. However, physical measurements, completed implementations and integration test results are still needed to demonstrate readiness. |
| What did AI miss that human discussion revealed? | Our group highlighted that minimising the number of bits introduces precision/accuracy trade-offs. Buddy 4 considered using 2mm steps for hump height to better match measurement uncertainty, while Buddy 5 emphasised maintaining sufficient ultrasonic precision for reliable obstacle profiling and avoidance. Our group decided to keep the separation RTOS tasks instead of redesigning the architecture solely to meet the new constraint. |
| What did AI correctly identify after discussion? | AI correctly identified missing binary field specifications, possible synchronization risks involving shared packed data, unverified measurement precision and the absence of quantitative memory and timing evidence. It also identified incomplete fault-handling rules and the need to validate encoded messages across subsystem boundaries. These gaps require further specification and testing before the team can demonstrate full integration readiness. |